We had a customer needing a step-down transformer for an ADC-66 today.  Square D is now owned by Schneider Electric and they are the manufacturer of the transformer.  These three-phase transformers are now required by our government to be Energy Efficient.  Therefore, the part numbers have changed.

ADC-44 use transformer number EE30T3H

ADC-66 use transformer number EE45T3H

For your customer to locate a Schneider Electric dealer in their area they can go to:

http://www.schneider-electric.us/sites/us/en/support/distributor-locator/power-industry-locator.page

Again, these are three-phase transformers that will step down 480-volts to 208-volts.  In addition, they offer a secondary 120-volt connection point to operate our 120-volt control circuit on our ADC-66.  On this machine (ADC-66 only) you will need to disconnect the internal 120-volt and reconnect it to the transformer supplied 120-volt.