ADS BULLETIN #02-44

Effective Date 8/94

Rack travel and counting with a timer in CONVEYORS

Rack-style conveyors move dishracks across the sprays by the reciprocation of articulating dogs.  These dogs push in one direction and fold down in the return direction.  They push against the "ladder" on the underside of the dishracks.  The "speed" of NSF listed conveyor dishmachines is determined by the length of time required for a Metro G-88 glassrack to travel across from the soil table to the clean table.  The starting and ending point is the table lip.  The indicating point is the trailing edge of the rack.  If it takes 36 seconds to cross this distance the rated speed will be 6 feet per minute.

The speed is determined by two items: the action of the conveyor dogs and the placement of bars on the rack ladder.  If either of these are missing or damaged the speed will drop.  If too many ladder bars are missing the rack will not travel through the machine.  In actuality, the rack is responsible for 50% of the conveyor's speed mechanism.  Racks other than the Metro G-88 glass rack will cause some variations in the speed.  The ADC-44 uses ten dogs to reduce this variation and give more consistent output.

If a clock timer is installed to act as a rack counter it can only record the amount of time the machine is operational.  It can not sense an actual rack.  It can, however, accurately record the amount of time the machine uses chemicals and energy.  By estimating the number of potential racks the conveyor is capable of processing you can arrive at an approximation.  Fortunately, the ADC design operates only when a rack is present and shuts down mechanically when the rack exits.  This will give closer rack figures then other styles of conveyor control devices.   If there is an inaccuracy in the overall count, it will be less actual racks washed then the multiplied figure.  Note: while the actual rack count may be less, the amount of chemicals, water and energy (over time) will be correct.

14

ADS BULLETIN #11-44

CONVEYOR DISHMACHINES

Effective date 8/22/94

Subject: RACK COUNTERS for conveyors

Action:  Clock timer (hours/tenths) wired into final rinse signal

Reason: Individual racks can not be counted by electro-mechanical means.  Sophisticated sensing devices have proven to be inaccurate and unreliable.  The problem stems from these fundamental circumstances:

A conveyor has no doors		Rack movement is continuous or single

Rack sizes, conditions vary	A timing device can be interrupted

Wash/rinse events intermix	Racks present a “wall” with no break

Recommend: We can accurately count the time the machine is in operation, or cycle length.  We call this a rack timer, but it can not identify a rack.  We simple count the amount of time the final rinse is in operation, then multiply the NSF rating of racks per hour by the actual hours on the rack timer.

Such as 3.5 hours x 244 = 854 racks (if all conditions are optimal)

Conditions of rack ladders, dogs, conveyor assembly, and procedure would change (less per hour) the outcome.  The number would fall from the optimum of 244--it would not increase.