Bulletin: HT-25 UPDATED CONTROL BOX

(230v control circuit, non-neutral, 7/6/12)

A request for ADS to remove the neutral requirement for electrical power supply in the HT-25 model has been answered with a conversion from the 120v control to a new 230v control circuit.  The main difference seen by installers will be feeder hook ups.  Instead of 120v signals of the dispenser hook up, the detergent and rinse terminals will now have 230v power.  Connect chemical dispensers accordingly.

 CAUTION

There are two live phase circuits (L1, L2) that make up the control voltage.  Care must be given to troubleshooting when hands come in contact with wires that can deliver 230 volt shock.  This is dangerous voltage that can be deadly.  The two live phases are fuse protected, but this will not protect a person who connects to both phases.  Only trained persons shall access or service the electrical controls.

HOW TO TELL THE DIFFERENCE FROM 5-WIRE NEUTRAL BOX TO 4-WIRE BOX

The control box layout will look different.  Instead of the cam timer being mounted in the upper left-hand side of the control box, it will now be mounted upright on the “floor” of the box.  This will make it easier to adjust and access.  Auto-start and heater solid state relays will be located on the upper left-hand side of the control box.  All contactors and electrical-mechanical relays, heater circuit breaker, control circuit fuses are mounted on the center din-rail.  Finally, the power distribution block will be mounted on the right-hand side of the box with (4) four terminal locations for L1, L2, L3, and Ground.  If there is a high leg in the building’s power (240v delta) connect that leg to L3 in the distribution block.

DISPENSER HOOK-UPS

The hook up for dispenser is located on the left-hand side of the box midway up the wall.  Rinse is toward the front, detergent is toward the back.  Detergent signal comes from the cam timer and can be adjusted for length of time on.  The signals are 230v power.

WIRES FOR CONTROL CIRCUIT

There are no white wires in the non-neutral control box.  L1 comes in as part of the control circuit on a black wire, L2 makes up the other half on red wires which collect at a 18-pin terminal strip located at the bottom of the left-side  wall of the control box.  L1 (black) 18-pin terminal strip is located just above the L2 (red).

INCOMING POWER CONNECTION

Incoming power wires connect at the main distribution block located on the right-hand side of the control box back wall.  There are (4) four connection screw terminals—L1, L2, L3, GND.  The power for the machine is 208-240v, 3-phase, 40 amp (50 amp for attached booster single point connection).  Eight gauge (8 AWG) is the minimum requirement.

Booster Heater (Installation note from heater manufacturer)

“For a 70° rise application static pressure should not exceed 50 psi.” If static water pressure exceeds those numbers, a Pressure Reducing Valve must be installed.  This pressure reducer comes before the booster.  In the event the reducing valve does not correct the pressure problem the following should be done:

1)      Look for and remove any check valves on the inlet supply line (the Hubbell operation and maintenance manual also clearly states that no check valves should be used during installation).

2)      Install a pressure reducing valve with a built in bypass on the supply line. Ideally the pressure going into the booster should be 20-25 psi, so the adjustment range of the PRV has to include the 20-25 psi range.

3)    Install a thermal expansion tank on the supply line.

The control box conversion to 230v requires some alternate control parts.  They are listed below:

Water solenoid   ADS PN 092-5068,

Cam Timer Motor—45 sec, 60 sec, 72 sec ADS PN 391-3014, 391-3011, 391-3015,

Double pole relay ADS PN 291-3029,

Timer Delay Relay ADS PN 091-3084,

Heater Contactor Delay Relay ADS PN 291-3031,

Pilot Light, System “On”  ADS PN 391-7104

Operation/cycle lamp ADS PN 391-7106,

Motor Controller, Wash Motor ADS PN  291-3033,

Wash Tank Heater Contactor ADS PN  291-3043,

Counter ADS PN 391-3013,