ADS Tech Information

Wash Tank Will Not Maintain Temperature

Conveyor Machines

Tank Temperature:

The NSF specification for MINIMUM Temperature of the wash tank is 160F for high temperature sanitizing.  The thermostats are set to points closed at 160F and points open at 165F (plus or minus one degree).  Verify the tank thermometer is reading correctly with other source.

The short story is this:

There are only three issues to determine a solution.

ONE

The only thing that will put heat back in the water after spraying is the kW of the heating element.  The heater has to be working on three legs, free from calcium build up, and be rated for the available voltage.

TWO

The only thing that will give the heating element kW is the voltage service.  Voltage levels vary widely among the 200v powers.  200-250v are seen in all areas of the grid (sometimes 190s during summer months or services that exceed capacity).  Example of low voltage on kW rating: 200v on a 220v, 12 kW heater will only provide 9.9 kW.  So, ADS offers a range of five wash elements, two of the higher kWs must be supplied with wire and circuit breaker larger than that found in the standard production conveyor.

THREE

Conditions at the account—they are all different.  Conditions that range from missing curtains, draining tables, poorly maintained equipment, undersized booster or heater, type of dishware (nursing home dinner trays draw off heat rapidly), procedure, and thermostat adjustments.

The only measurement that is relevant is the tank temperature while the pump is spraying water (after equalizing 3-5 minutes, temp should come up to the thermostat set points and hold—if not see one of the above).

Russ Payzant