Overhead Hoods verses Pant Legs

Yes, hoods treat the room—the space for people.  They can effectively ventilate and cool the people spaces.

Pant legs are ducts that ventilate and cool the machine’s air—the space for the cleaning process.  We hardly ever find a pant leg duct balanced or effective (meaning getting rid of unneeded temperature from the space for people vs. allowing maximum use of heat in the cleaning process).  They are only promoted and defended by the cost of installation.  We find very few HVAC professionals who understand the difference, consequently the myth that “there is no difference and pant-legs are so much cheaper.”

Hood: high one-time cost, less impact on daily cost of cleaning processes of the dish room

Pant leg: cheaper install costs, higher daily energy costs of the ware-washing equipment.

Under no condition is it recommended that a pant leg be extended up into a hood and open to the hood volume.  A pant leg must be ducted to a fan source to draw air.  The expectation that the pant leg will somehow “chimney” the hot air up to the hood is like placing a straw on the table and expecting the spilt milk to be sucked back into the straw.  It seems that a cheaper price tag will cause some people to accept the absurd . . .