HT 25 Tech Support

Hello fellow metal pushers.

I promised to keep you informed, if something came along that was a concern and here it is, for this week.

It concerns the HT-25. They are suppose to be pulling the cube relay, when the door start relay is installed, which is every machine now. This is the small plastic relay that set's beside the wash motor contactor. As you know, we have been pulling the cube relay out of low temp machine for many years, when they order door start.

I had two calls within one hour, concerning this issue. They had left the relay in place and after a month or so it will begin to chatter, then cause the contactor to chatter. The simple fix , have them pull it out and lay it in the bottom of the control box, to save it for the day the door start relay does go out, which they do on a fairly regular basis. One thing you need to bring up in your next FFF meeting is the failure rate of the door start relay.

I have talked to Dan about this and they will be pulling this cube relay out, but if they miss one you now know what to tell your dealers, who call with a chattering relay.