AMERICAN DISH SERVICE

TECHNICAL SUPPORT MATERIAL

ADC – 44/66 Final Rinse Water

The final rinse portion of the conveyor comes from the building’s fresh water supply.  It is sprayed from the final rinse manifold and does not come in contact with any of the tank sprays.

The water curtain design (mistakenly referred to as pumped rinse) provides a shielding spray of pumped water before the final rinse.  The water comes from a holding tank located under the final rinse manifold and is replenished by final rinse water after it is sprayed.

NOTE: the water curtain is pump-maintained at 3 PSI and has the force of water pouring out of a glass in the upper spray arm.  The lower arm should raise only 4-6” above the arm.  The only purpose of the lower arm is to shed soapy water off the bottom of the dishrack.

Rationale: If the sprays from the lower arm rise higher, they will overflow into the wash tank and empty the curtain’s reservoir.  This becomes a serious problem to machine operation.  For this reason, the water curtain sprays must be check in routine maintenance for straight up/down direction and no higher than 4-6” above the bottom arm.

Some Detergent carry-over is expected with-in the machine.  Final rinse, however, is not open to the machine tanks or sprays.  At 20 psi, the final rinse will rinse sprayed wash detergent from the dishware.  This can be verified by the Pheno test.