Conveyor Speed and racks per hour for

ADS Conveyors

ADC-44/ADC-66  Speed 6.8 Ft/Min., GPH 120.

Racks per hour:

NSF/ANSI Standard 3 Formula (CS) (60) / RL

6.8 x 60 / 1.6666 = 244.897 racks per hour

Gallons per rack:

NSF/ANSI Standard 3 Formula (GPH) / (RPH)

120 / 244.897 = 0.49 gallons per rack

NSF/ANSI STANDARD 3
Commercial Warewashing Equipment

Racks per Hour / Dishes per Hour / Water Consumption Calculations

For informational purposes, the estimated racks per hour or dishes per hour for NSF/ANSI Standard 3 Certified dishwashers can be calculated as follows:

Racks per Hour for Stationary Rack Dishwashers = 3600(NR) / TC

Racks per Hour for Rack Conveyor Dishwashers = (CS)(60) / RL

Dishes per Hour for Rackless Conveyor Dishwashers = (120)(CS)(CW) / PD

Water Consumption (gallons per hour) for Stationary Rack Dishwashers = (GPC)(RPH)

Water Consumption (gallons per hour) for Conveyor Dishwashers = (GPM)(60)

Where: CS = Conveyor speed measured in feet per minute
              CW = Conveyor width measured in inches
              GPC = Gallons per cycle
              GPM = Gallons per minute
              NR = Number of racks the dishwasher is designed to hold per cycle
              PD = Distance between pegs in the direction of conveyor travel measured in inches
              RL = Length of the rack measured in feet
              RPH = Racks per hour
              TC = Total cycle time measured in seconds

The total cycle time includes wash, rinse, dwell, and load times. Load times will vary depending on the type of dishwasher. Estimated load times commonly used for specific types of dishwashers are as follows:
door type (straight thru) - 5 seconds
door type (corner) - 7 seconds
hood type - 8 seconds
front opening type - 30 seconds

| AMERICAN DISH SERVICE |  |  |
| --- | --- | --- |
| 900 BLAKE STREET |  |  |
| EDWARDSVILLE, KS 66111 |  |  |
| 913-422-3700 |  |  |

Facility : EDWARDSVILLE, KS

CONVEYOR DISHWASHING MACHINES

|  |  |  | Final Rinse |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (20 psi) |  | Conveyor Speed |  |
|  | Model | Conveyor | Flow | Usage | (Max.) | Footnotes |
|  | Number | Width | gpm | gph | (ft/min.) |  |
| Multiple Tank Rack Conveyor Dishwasher - Chemical Sanitizing |  |  |  |  |  |  |
|  | ADC-44 | 20" | 2.0 | 120 | 6.8 | [1] [2] |
|  | ADC-66 | 20" | 2.0 | 120 | 6.8 | [1] [2] |
|  | [1] Unit accepted when operated as a chemical sanitizing dishwasher provided it is installed with a Beta Technology Model H9200SR or H9300S chemical sanitizer feeder. [2] Unit accepted when operated as a chemical sanitizing dishwasher/glasswasher provided it is installed with an American Dish Service model 64-0001 or 64-0010 chemical sanitizer feeder. |  |  |  |  |  |
| Multiple Tank Rack Conveyor Dishwasher - Hot Water Sanitizing |  |  |  |  |  |  |
|  | ADC-44 | 20" | 2.0 | 120 | 6.8 |  |
|  | ADC-66 | 20" | 2.0 | 120 | 6.8 |  |