Tech Support for B13 Thermostat

The question came up concerning adjustment of the thermostat on the B-13.  While no one at ADS wants to see customers set the thermostat beyond the boiling point, which will cause blow-off and element damage—it is still a good question.

To answer:

We informed UL through Brian Warner in Conformity Assessment on 9/11/01 that we would have a maximum setting on the thermostat of 190F for the pressure vessel.  Further, in the UL Preliminary Investigation Letter of 2/26/02, we were told that we must show that we comply with temperature-limiting control (UL # 353) and temperature regulating controls (UL #873).  We had to send a sample or drawing of how we intended to limited the setting on the thermostat.  Since the thermostat we purchase does not have a set screw to lock the adjustment rod, we had to find another way. This is when I made the drawing of a cover plate to attach over the thermostat so that it could not be inadvertently adjusted beyond 190F.  It was later decided to remove the plate and just block the hole with the cover itself.

This was accepted with my letter of 3/6/02 to Brian Warner along with answers to ten other conformity issues.   I hope this will clear up the reason why the thermostat is mounted behind the inside cover.  For everyone’s information, from 190F to 212F is a very small movement of our thermostat dial.  The resulting temperature climb to the boiling point does not happen immediately and so it is hard for the customer to associate their actions with the ultimate result of heater failure.  When this happens, the evidence is all three heaters burn out at the same time.

Best regards,

Russ Payzant