AMERICAN DISH SERVICE

# TECHNICAL SUPPORT MATERIAL

# Routine and Preventative Maintenance

ADS PUMP SEAL #092-2001 INSTALLATION

Showing the pump housing #082-6301, motor side           Pump housing, pump side

Use liquid soap to lubricate seal bore                                   Ceramic, install with rubber boot on bottom      Installing tool #099-6007 to press to bottom of bore        Inspect back side, seal sits flush to metal

Install motor to housing, caution of ceramic                       Black smooth face goes on motor shaft next

Press  black portion of seal w/ installing tool                        Press seal so 1/8” below the threads

Place spring on w/spring cap toward impeller                      Screw on impeller by hand, rh threads

IDENTIFYING IMPELLERS AND PUMP COVERS

Standard closed face #082-6303(AF-3D-S, ET-AF, 5AG-S)      Open face #082-6306, (L3DW-S)

Open-Modified face #, (AF-ES, 5AGES)                                     O-Ring gasket pump cover #082-6307 for open impellers

Standard, cork gasket, pump cover #082-6302 for closed impeller