?   ADS ?

ESSENTIAL

INSTALLATION  INSTRUCTIONS

FOR

ADC-44/ -66 RACK CONVEYOR DISHMACHINE

7/28/00

Failure to follow manufacturer's installation requirements will void warranty claims and may result in charges for follow-up services

Manufacturer's Recommendations:

#1, Prior to the Installation, arrange for necessary assistance from electricians to run appropriate wire.  In the case of wire gauge change or Neutral supply wires, have the requirement satisfied before the install or arrange for an attending electrician.  The dishmachine shall not be installed contrary to NEC or the mfr's specifications.

#2 Inspect the site before the installation for table conflicts and drain problems.  NO TABLE DRAINS (quickdrains) or sinks within 20" of dishmachine on either table, and always use 2" pipe, few 90 degree elbows, drain downhill.  Failure to observe these two recommendations may nullify warranty claims.

ELECTRICAL for 44 3 Phase

1.  Provide a 3 phase, 208 volt, 60 amp, clean circuit to the machine.

2.  The power supply must consist of three 6 AWG wires and a ground wire.

3.  A conduit hole size of 1" is provided in the control box for the electrical service.

4.  Attach the heavy gauge wire in the main distribution block on the left side of the control box.   Make sure these connections are tightened with an Allen wrench.  Failure to do this can result is burned wires.  If you have a 230 volt, or delta system, attach the high leg (200v line) to L3.  Attach the other two lines (110v lines) to L1 and L2.  If motor rotation is going the wrong way (check arrow on conveyor gearbox) exchange the incoming power lines at L1 and L2.

5.   CAUTION.  Before powering up the machine check all screw lug terminals in the control box and the heater power box (located under the rinse tank).  These screws can loosen as a result of temperature and wire flexing.  A loose connection at a high amp terminal can cause burning of components over time.

ELECTRICAL for 66 3 Phase

1.  Provide a 3 phase, 208 volt, 90 amp, clean circuit to the machine.

2.  The power supply must consist of three 3 AWG wires and a ground wire.

3.   A Neutral wire (10 AWG) must be supplied for control circuit

FRESH WATER PLUMBING

1.    In-coming water connection: (ALWAYS FLUSH THE LINE BEFORE CONNECTING TO A NEW MACHINE)  Chemical sanitizing requires a 3/4" FPT at 120F minimum, 20 psi at flow.

2.    Hot water sanitizing requires 3/4" FPT for 160F fill water and 1/2" FPT with a pressure regulator.  This 1/2" line is for the 180F (minimum allowed) final rinse; it also requires 20 psi during flow.  The center adjustment screw on the pressure regulator turned clockwise will increase pressure.

3.  The mixing chamber in the final rinse manifold has two open ports for the dispensing of chemicals.  These ports will need check valves from your dispenser.

DRAINS

1.  The machine comes with a copper, 2" common drain.  The exit for this drain is on the soil side of the machine.  When you attach a drain to the machine, keep in mind that approximately 15 gallons of water dump out when the drains are pulled.  Floor sinks used as conveyor drains will be a source of flooding and irritation for the account.

The drains should be as straight as possible.

The use of 90 degree and 180 degree bends will cause clogging and service calls.

Drains must go downhill.

TABLES

The greatest source of service calls for conveyor accounts come from incorrect table application.  Not all table types are suitable for conveyors.  The conveyor has no doors and must operate with either side open.  Water can be lost due to table construction and placement of sinks and drains.

1.  It is recommended that sinks and drains be a minimum of 20" from the edge of the table lip.  If a drain is closer than 20" it must be removed or a new table installed.  These types of tables are for door-type equipment and harm the operation of conveyors.  This is best considered before the installation.

2.  The tables must slant toward the machine so water can return to the tank.  Otherwise, the water will be lost and the machine's design for self-scraping will be canceled out.  This will also cause the auto-fill to activate repeatedly, thus increasing water and chemical usage.

The economical answer is this: the tables are made to serve the machine's needs, not the reverse.

3.  When the tables are finally put in place, ANCHOR the table to the dishmachine using the 1/4" -20 SS hex bolts supplied in the machine package.  There is a diagram for the position of the bolts along with two new drill bits.  This is important, if you anchor the tables you will eliminate many service problems.

4.  The table lip should not extend down into the machine far enough to rub on the conveyor bar.  If this should happen cut a half-round in the table lip above the conveyor bar for clearance and fasten down the table securely on the machine.  There should be no space between the table and the machine's lip.

Reason:  If the elevation of the table were raised above the level of the tank lip, then the geometry of the conveyor dogs, switches, and racks will be changed.  To compensate for a raised table several items must be altered and adjusted.  This will create a major field modification to the 	machine's control and conveyor systems.  It is recommended that the tables simply be attached to the machine according to these instructions.

TABLE LAYOUT

A 90-degree exit or a "horse shoe" layout are popular ideas because of the space saved but they cause costly problems.  Even with the safety clutch the difficulty of moving racks around a corner is extreme.  In most cases a table limit switch will be required to shut down machine operation.  This layout reduces a conveyor's output to nearly that of a corner machine.

THE RULE FOR ALL DISHMACHINES:  The Installation is KEY to reduced service problems and complaints.

DISPENSERS

1.  The final rinse manifold has two ports (1/8" IPS, female) provided in the mixing chamber.  The dispenser's electrical connection points are provided in the control box.  There are two signals.  A 208/240v signal labeled RINSE and 208/240v signal labeled DETERGENT.  They are located in the lower left hand area of the control box.  The 110v CONTROL (1amp) circuit produced by the transformer does not have the capacity to run a dispenser.  It is highly recommended that no device be attached to the control circuit of the machine, such as dispensers and hood fans.  Use the factory provided dispenser terminals in the control box.

2.  The wash tank has a 7/8" hole provided for a probe.  It is located at the front of the tank adjacent to the pump filter and pump discharge manifold.  There is also a 7/8" hole for the detergent inlet located in the wash section above the scrap trays on the side of the well between the wash and rinse tanks.  This can be accessed behind the heater power box located in front of the rinse tank pump.

3.  Do not mount the dispenser on top of the control box.

4.  Secure all chemical lines and attachment points.  Make sure that chemical lines do not run over the top of electrical devices or inlet plumbing.  (a leaking sanitizer line can destroy the stainless cabinet and machine components)

MACHINE OPERATION

After the machine is operational and the tables are cleared, remove all packing material (heater supports) then wash out the tanks, give attention to any installation debris left over.  Now, fill the machine by turning on the master switch.  The Conveyor will automatically fill with water to an operational level.  IMPORTANT: Adjusting the water diverter plate may be required to fill the tanks in proper sequence.  It is found inside the hood on the back wall at the top.  Slotted holes allow movement side to side.  The rule is: make sure the rinse tank always fills last.  After the pump motors start, the machine will fill once more to compensate for the water used by the pumps.  This is normal operation.

Place a rack at the opening of the soil side of the machine and push it in until it catches on the first dog.  The machine will take the rack through, operate the pumps, conveyor, heaters, final rinse and then exit the rack and shut down until another rack is put into the machine.

WHAT TURNS THE MACHINE ON AND OFF

The conveyor motor, wash motor and rinse motor are all sequenced by the actual dish rack.  There is no timer or control unit that causes the machine to run.  Everything is keyed off the dish rack.  The sequence bars located in the front tray track are used to tell the machine what to do.  The reed switches are normally closed (they always TURN ON THE MOTORS), when a magnet comes within ½" of the reed, it will turn OFF.  When the magnet swings away from the switch, the motor turns on.  NOTE: an easy way to test the reed switch is by placing a hand held magnet next to the reed, if it turns off that means the switch is good and the problem is alignment with the machine's bar magnet.

The heaters are controlled by the thermostats and the level control switches.  If there is no water in the tanks when the machine is turned on, the level control switches will turn off the heaters and open the fill solenoid.  When the tanks are full the switches will shut off the water and then turn on the heaters.  If the incoming water is hotter than the settings on the thermostats, the thermostats will shut the heaters down until the water temperature drops below the set point.  NOTE: To increase the temperature set point, turn the thermostat center rod counter clock-wise.  (turn to the left)

The dish rack acts as 1/2 of the conveyor mechanism.  The center dogs pull against the ladders of each rack.  If the rack or its ladder is damaged the conveyor system will work less effectively.  Damaged or broken racks should be replaced.  They cause problems such as racks staying in the machine or riding back a forth on the conveyor bar.  Both of these problems will cause excessive water, chemicals, and energy use.  The rack is 1/2 of the system.

A test to see if the machine's tray track and tables are set up right is to place a rack on the soil table then push it through the machine and on to the clean table.  It should slide easily across all points.  If the rack should stick between the two tray tracks, they will need to be moved wider apart.  The track is attached at three points, adjust all three to maintain alignment.  (Never adjust the front tray track; always make the adjustment to the rear tray track)

CAUTION:

Always disconnect power to the machine before servicing.

Beware of moving gear and drive arm on conveyor.

Heaters will remain hot immediately after emptying of machine.

Turn machine off before opening for inspection.

The complete Conveyor Service Manual (78 pages) is available from ADS and can be ordered from the home office at 800 922 2178.