LOW TEMPERATURE MACHINES

There are three basic styles of low temperature dish machines. They are the single rack, the double rack and the conveyor. There are also a few under the counter machines in the marketplace for either smaller accounts or for the bar glass washers.

SINGLE TANK MACHINES: These machines are similar in appearance to the single tank high temperature machines. There are a few manufactures that manufacture a recirculating wash water machine with a sanitizer injected in the final rinse. The more prevalent type of low temperature single rack machine is the batch type machine. This machine usually holds less than two gallons of water. Detergent is added during the wash cycle and the entire wash water is drained at the end of the wash cycle. Fresh water is then added along with the rinse agent and a chlorine sanitizer. After the rinse cycle is run, this becomes the wash water for the next cycle. The advantage of this type of machine are, that you are always washing with clean water and that you are washing with a higher pressure. The rinse pressure is also always constant since it is a pumped rinse. The detergent pumps are normally built into this type of machine.

The specifications for this type of machines are as follows:

WASH TEMPERATURES	Even though the health departments will accept 120 degree water, you should always use 140 degree water to insure proper results.

FINAL RINSE	Again, even though the health departments will

TEMPERATURES	accept 120 degrees you should use 140 degree water to get spot free dishes.

WASH PRESSURE	Normally around 17 psi

RINSE PRESSURE	The same as the wash pressure since it is pumped by the same pump and does not come from the fresh water supply.

CYCLE TIME	Normally 90 seconds, but 60 and 120 second cycles are available.

SANITIZER SETTING	Normally a chlorine sanitizer is used and is injected at a rate equal to 50-200 parts per million.

DOUBLE RACK MACHINES:	These machines are basically the same as a single rack batch machine except that they hold two racks of dishes. The amount of water in the wash tank is about twice as much. The specifications are the same.

CONVEYOR MACHINES		This machine resembles the multiple tank rack conveyor in that it has multiple tanks: but. They are much smaller. The final rinse is a fresh water rinse with chlorine injected. The specifications are basically the same as the batch machine.