76

AMERICAN DISH SERVICE

TECHNICAL SUPPORT MATERIAL

SUBJECT: TUNING OF BATCH TYPE,

.	DUMP & FILL DISHMACHINES

The machine type known as low-energy, dump & fill, batch type, low-temp or LED are all equivalent terms for the NSF classification for chemical sanitizing dish machines. The category's principle elements are: chlorine sanitizing. Circulated pumped rinse, draining of entire batch during wash cycle. and high pressure / flow spray. The differentiating characteristics to typical (or hot water sanitizing) machines are: no holding tank, no heaters, and no final rinse.

Ordinarily, the batch-type dish machine is prone to have "result" problems. These are interpreted as spots on glassware, streaks on flatware, soil on dishware, wet dishes and trays. All of these problems are associated with the tuning of the machine and the water supply. .

The water coming to dish machines will vary in both temperature and pressure. There is no realistic way to standardize the primary water supply. It must be treated at each site. Generally, the heating source will be undersized. This means the hot water will run out before the recovery rate allows the restoration of the correct temperature during continued use of the dish machine. In addition, the water heater may be operating below its normal efficiency or be set on a lower temperature. Distance to the machine is also a factor; forty feet or more will present consistency problems. For a machine capable of 37 racks per hour, the required water heater recovery rate should be 80 gph. This is for the dish machine alone; any additional requirements would increase the rate. The ideal temperature for a batch-type machine is 130-140 F. Temperatures below 120 or above 150 create result problems, problems you cannot correct with more soap or more water.

The impelling force of potable city water is measured by two values: Flow and Pressure (head). Flow is the amount of water delivered over a period of time and is determined by pipe diameter. Pressure can be compared to the force applied to that delivery. A clogged 1/2" water line can have high pressure but deliver a small amount. Conversely, an open 3/4" line with low pressure will still deliver a small amount of water. . High pressures (70

90psi) can cause control device and joint problems, very low pressures (20psi) will take longer cycle times to properly operate the equipment. Pressure regulators can assist with the reduction of high pressures and pressure bladders or reserve tanks can help some low pressure problems.

The information above is background, now we will cover the tuning of the machines.

The first step is to set the correct water usage. There are only two elements in this procedure. One is fixed (drain earn), meaning you cannot adjust or modify and the other is adjustable (fill cam). The fixed part is called the drain cam. The drain cam establishes the sequence of the cycle and is required by NSF. When the cycle begins, the motor turns on and immediately the detergent cam operate, but do not adjust this cam yet. After some seconds of wash (45 sec.) the drain cam rotates into position and the switch.

77

Finger drops down on the low cam. This energizes the drain solenoid and causes all the water to exit into the drain.

IMPORTANT: During this time, all the water must exit the machine. About 10 seconds into this sequence you should hear the typica1 hollow running sound of the wash pump. If you do not hear this sound the machine still has soiled water in the system. The reason will be too much water coming into the machine. Reduce the incoming water by time (the fill cam is. the Adjustable part of the procedure). Decrease the space on the fill cam so the water solenoid is closed sooner. Now, run the machine again and see if all water is dumped from the machine-you'll also hear the pump running dry. As soon as you bear that sound (>) you can move the water cam so that the "fill" begins immediately. This will help purge the machine and start recharging the pump.

The next part is the adjustable tuning of the machine or setup. Connect a pressure gauge (kit #88-1048) to the lower spray arm. Run the machine again and watch the pressure build up after the drain closes. You should see the needle make three distinct Surges up to 17-19 psi and then hold steady at that pressure. This point (*) is called full spray arm pressure. This is the point when you can close off the water fill cam. Now the machine will operate within the best conditions for that account. Run the machine through another complete cycle. You should see steady pressure during the wash. A drop to zero during the drain, three surges back to full spray arm pressure (after the fill sequence), and full . pressure during the remainder of the rinse. This is the normal profile for performance.

DRAIN CAM:	\ drain > 1	1

WATERCAM:	-	-\fill	*/	1

TROUBLESHOOTING

If you see cavitations or wide .swings of the pressure gauge during the wash or rinse cycle it is caused by too little water. Increase the water. If you see a lack of any significant pressure during the rinse cycle but good pressure during the wash cycle, then the temperature is too high or the pressure is too low to recharge the pump. You can use the "burp-wire" procedure to correct these problems) If you see the normal profile but the pressures run 15, 12, 10, or 8psi then some or all of the impeller vanes are blocked. We will assume the drain screen is clean and the inlet manifold is clear.

There are two problems that are difficult to solve and there are few alternatives. One is fluctuating water pressures. When there are wide swings in pressure (10 - 40psi) there

Will be starvation or overfilling no matter how you set up the machine. Batch-type machines are more sensitive then final rinse machines. This condition is caused by inconsistent layout of the building's water system and demand fixtures. The other problem is water condition itself. Some elements that make up potable water supplies are complex and difficult to compensate with the basic ware washing formulas.

78

SPRAY ARM PRESSURE  TESTER                                                      For Batch~Type, Dump&Fill Dishmachines

(ADS Kit #88-1048)

Reading of “0”. When pump is stopped

When the pump starts with water in the tank. Normal pressure should read approx. 19psi.

KEY

Recovery has three typical surge patterns

Before normal pressure returns:

The pattern of needle movement tells a story of what is taking place in the pump dynamic.

The needle pattern will tell you:

1. Is the impeller dogged?

2. Is there enough water?

3. Is the water too hot?

4. Is the wash arms wore out?

5. Is there foam in the system?

6. The point of pump recovery

A blocked or clogged impeller will show the same recovery patterns but overall pressure will drop in 2psi increments