ADS Tech Support

ADJUSTING DRAIN BALL & SOLENOID

Drain all the water out of the machine and put the drain tube and ball
in the machine, resting on the drain seat. Tap the top of the drain
tube. It should lean no more than 1/2" in any direction. (From straight
up) If it leans more adjust the drain solenoid up. If  it swings back and
forth adjust the drain solenoid down.