American Dish Service

Delime Switch Instructions

1.         Drill ½" hole in Control box and mount switch in hole. Mount switch close to the mercury relay.

NOTE: Mount switch completely inside control box so that access to switch can only be obtained by removing control box cover

2.         CONNECT WIRES;

A. Brown wire to bottom of mercury relay with existing brown wire

B. Red wire to top of mercury relay with existing red wire

C. Single white wire to the bottom of mercury relay with existing white wire

D. Remove existing white wire, connected to chemical pumps and timer motor, then tape them off.

E.  Connect white daisy chain wire to chemical pumps and timer motor, replacing wires removed in step D

3.         Place sticker in a location that serviceperson will see before using switch

NOTE; Now the installation is complete, check switch. In one direction machine will operate normally. In the other direction, chemical pumps will be bi-passed and recirculating pump will come on.

CAUTION. Do not add delimer chemicals to machine before all other chemicals are out of the machine. Do not add any other chemicals to machine until after deliming chemicals are out of the machine.

: